Polarimeter Spares:
Monochromatic light source: sodium lamp (589.44nm)
Sample tube: 200mm, 100mm
Power supply: 220V±22V, 50Hz±1Hz
Lamp power supply  card part no WZZ

Leak Test:
Solenoid valve ELT Series, 3 Station Manifold, 2/2 Single Solenoid Controlled, Normally Closed, Non-locking automatic Override, 1/2" In, 3/8" Out, BSPP, 24 V DC, programmer circuit TTL connection feedback, vacuum sustain -760mmHg.

Spares for Tablet Friability Tester
Opto coupler : 
Opto-isolated SCR devices silicon-controlled rectifier (SCR) detector.   step-up and slow to medium  switching speeds. Count time 1 sec to 99hr 59 min with programmable interface. PART NO. 1009125 MADE IN JAPAN

NAME OF THE EQUIPMENT:: STABILITY CHAMBER

. Capacity 400 Ltr 2. M.O.C SS – 304 3. Finishing GMP Model:Inside S.S 304 mirror polished & Outside S.S 304 dull smooth buff. 4. Conditions as per ICH Guidelines ---- 25 C & 60% 30 C & 65% 40 C & 75% 5. Temperature Accuracy ± 0.2 C 6. Temperature Uniformity ± 2. C 7. Humudity Accuracy ± 2% 8. Humidity Uniformity ± 3% 9. Feature Microprocessor based PID controller (Auto Tuning Type) with digital display of temperature in deg. Centigrade giving accuracy of ± 0.20 C. Printer Interface facility to print records on printer with set values, actual values, date and time with variable print time interval PC Interface communication card with RS 485 for above control system. 8point temperature humidity scanner with printing facility